Service Providers Accessibility Checklist

Location:

| A- APROACH AND ENTRANCE | Answer |
| --- | --- |
| A.1 Is the entrance of the building located at less than 50mt from the street? | Don’t Know |
| A.2 Is the entrance of the building located at less than 50mt from the parking lot? | Don’t Know |
| A.3 Parking |  |
| A.3.1 Are there enough parking bays reserved for people with disabilities? | Don’t Know |
| A.3.2 Are the reserved parking bay located at less than 50mt to the building entrance? | Don’t Know |
| A.3.3 Are the reserved parking bay min 3.9m x5.40m? | Don’t Know |
| A.3.4 Is the surface of the parking stable, flat and negotiable (no gravel, no sand, no obstacles)? | Don’t Know |
| A.3.5 Is there signage (international standards) marking the accessible parking lot so that it can be seen even when the car park is full? | Don’t Know |
| A.3.6 Is there any vehicle drop off area (min 9.00mts x 3.60mts) near the main entrance of the building? | Don’t Know |
| A.3.7 Are there curb ramps close to the accessible parking bay and/or drop off area to allow wheelchair users to go from the street level to the sidewalk level? | Don’t Know |
| A.4 Route |  |
| A.4.1 Is the surface of the route from the parking/drop off area to the building entrance stable, flat and negotiable (no gravel, no sand, no obstacles)? | Don’t Know |
| A.4.2 Are there street lights close to the building entrance, on the sidewalk and on the parking lot? | Don’t Know |
| A.4.3 Is there tactile pavement on the route from the street/parking to the building entrance? | Don’t Know |
| A.4.4 Is there any obstacle in the route? If so, are they clearly visible and detectable with a cane? | Don’t Know |
| A.4.5 The headroom along the route is maintained at a height of not less than 2 100 mm above the surface of the path? | Don’t Know |
| A.4.6 Where the route has differences in height, are the ramps and stairs in place? | Don’t Know |
| A.4.6 External Ramps |  |
| A.4.6.1 Is the surface width of the ramp/path minimum 120cm, ideally 150cm or 180cm? Is the unobstructed width of the ramp from handrail to handrail minimum 100cm? | Don’t Know |
| A.4.6.2 Is the length of end and intermediate landing at least 150cm? | Don’t Know |
| A.4.6.3 Does the ramp have an acceptable gradient? | Don’t Know |
| A.4.6.4 Does the ramp have guarding kerbs? | Don’t Know |
| A.4.6.5 Does the ramp have an anti-slip surface? | Don’t Know |
| A.4.7 External Stairs |  |
| A.4.7.1 The rise of the step is higher than 150 mm, and the going of the steps are not less than 300 mm? | Don’t Know |
| A.4.7.2 Is the minimum width of the stairs 120cm? Is the minimum width between handrails 100cm? | Don’t Know |
| A.4.7.3 Is the rise of the step closed? | Don’t Know |
| A.4.7.4 The flight of steps contain maximum 16 risers? | Don’t Know |
| A.4.7.5 Is the minimum width of the landing 150cm? | Don’t Know |
| A.4.7.6 Is the clear height under stairs minimum 210cm? | Don’t Know |
| A.4.8 Handrails |  |
| A.4.8.1 Are handrails located on both sides of the ramp and stairs? | Don’t Know |
| A.4.8.2 Is the handrails profile diameter max 4.5cm and minimum 3.5 cm? | Don’t Know |
| A.4.8.3 Are the handrails located at least 4cm from the adjacent wall or other obstructions? | Don’t Know |
| A.4.8.4 Are the first set of handrails located between 85cm and 100cm? Are the second set of handrails located between 60 and 75cm? | Don’t Know |
| A.4.8.5 Are handrails horizontally extended for at least 30cm at the begging and end of the ramp/stairs? | Don’t Know |

Comments:

Photos of Section A :

| B- ENTRANCE | Answer |
| --- | --- |
| B.1 Is the entrance to the building marked and visible? | Don’t Know |
| B.2 Is the main entrance accessible? If not, is there an alternative accessible entrance that can be used independently and in the same hours of the main entrance? | Don’t Know |
| B.3 Is the main entrance leveled? If any threshold is in place, the height doesn’t exceed 20mm? | Don’t Know |
| B.4 Is the unobstructed width of the entrance doorway at least 90cm? Is the unobstructed height of the entrance doorway at least 200cm? | Don’t Know |
| B.5 Is the horizontal maneuvering space in front of the entrance door of minimum 150cm by 1 50cm? Is there a clear space of 60cm (70cm recommended) at the latch side of the door to allow someone to operate the door handle while using a wheel chair? | Don’t Know |
| B.6 Is the door equipped with a handle that is operable with one hand and does not required tight grasping, pinching, or twisting of the wrist? | Don’t Know |
| B.7 The carpets or mats at the entrance of the building are less than 12mm? Are the edge of the carpets or mats securely attached? | Don’t Know |
| B.8 Is the entrance to the building marked and visible? | Don’t Know |

Comments:

Photos of Section B:

| C- ACCESS TO SERVICES | Answer |
| --- | --- |
| C.1 The accessible entrance provide direct access to the main floor and reception? | Don’t Know |
| C.2 Are all public spaces on at least one accessible route? | Don’t Know |
| C.3 Is the surface of the routes stable, flat and negotiable (no gravel, no sand, no obstacles)? | Don’t Know |
| C.4 Is the unobstructed width of the routes minimum 120cm, ideally 180cm? | Don’t Know |
| C.5 If 90 degrees wheelchair turns are required, a minimum maneuver space of 120cmx120cm, ideally 150cm x 150cm is provided? | Don’t Know |
| C.6 If 180 degrees wheelchair turns are required, a minimum maneuver space of 200cm x 150cm is provided? | Don’t Know |
| C.7 Vertical circulation |  |
| C.7.1 Internal Stairs |  |
| C.7.1.1 The rise of the step is higher than 150 mm, and the going of the steps are not less than 300 mm? *Exceptionally on adaptations of existing buildings the minimum going of the tread shall be 260 mm, and the maximum rise shall be 180 mm. | Don’t Know |
| C.7.1.2 Is the minimum width of the stairs 120cm? Is the minimum width between handrails 100cm? | Don’t Know |
| C.7.1.3 Is the rise of the step closed? | Don’t Know |
| C.7.1.4 The flight of steps contain maximum 16 risers? *Exceptionally when the plan area is restricted, a flight of a stairs shall contain no more than 20 risers. | Don’t Know |
| C.7.1.5 Is the minimum width of the landing 150cm? | Don’t Know |
| C.7.1.6 Is the clear height under stairs minimum 210cm? | Don’t Know |
| C.7.2 Handrails |  |
| C.7.2.1 Are handrails located on both sides of the ramp and stairs? | Don’t Know |
| C.7.2.2 Is the handrails profile diameter max 4.5cm and minimum 3.5 cm? | Don’t Know |
| C.7.2.3 Are the handrails located at least 4cm from the adjacent wall or other obstructions? | Don’t Know |
| C.7.2.4 Are the first set of handrails located between 85cm and 100cm? Are the second set of handrails located between 60 and 75cm? | Don’t Know |
| C.7.2.5 Are handrails horizontally extended for at least 30cm at the begging and end of the ramp/stairs? | Don’t Know |
| C.7.3 Elevators |  |
| C7.3.1 Is the minimum inner dimension of the car 110cm x140cm? Is a minimum unobstructed entrance width of 80cm (ideally 90cm) provided? | Don’t Know |
| C.7.3.2 Is there at least one horizontally fixed handrail in the car, on the same side as the car operating panel? | Don’t Know |
| C.7.3.3 Are controls placed at a minimum distance of 500 mm (600 mm recommended) from any adjacent corner or wall and between 900 mm and 1 200 mm above floor level, preferably 1 100 mm? | Don’t Know |
| C.7.3.4 Are there Braille or tactile figures complementing the information in the control panel buttons? | Don’t Know |
| C.7.4 Wayfinding and Signing |  |
| C.7.4.1 Are there tactile surface indicators in place (attention pattern and guiding pattern)? | Don’t Know |
| C.7.4.2 Is a wayfinding system provided (map/tactile map of the facility, routes indicators, audio guidance)? | Don’t Know |
| C.7.4.3 Are there signs designating rooms and spaces (room number, room? | Don’t Know |
| C.7.4.4 The characters of the signs contrast with the background? | Don’t Know |
| C.7.4.5 Are the text character raised? | Don’t Know |
| C.7.4.6 Are signs mounted on the wall on the latch side of the door? | Don’t Know |
| C.7.4.7 Are signs mounted between 120cm and max 160cm from the floor? | Don’t Know |

Comments:

Photos of Section C:

| D- TOILET ROOM | Answer |
| --- | --- |
| D.1 Is there at least one accessible toilet room? | Don’t Know |
| D.2 Is there a sign that give directions to accessible toilet rooms? | Don’t Know |
| D.3 Is there a sign at the entrance of the accessible toilet room with the International symbol of accessibility? | Don’t Know |
| D.4 Is there an outward opening door with minimum width 90cm? Is there an horizontal pull handle at 70cm height? | Don’t Know |
| D.5 Is there a clear maneuvering space of 150cm x 150cm inside the toilet? | Don’t Know |
| D.6 Is there a clear space of min 90cm beside the toilet (120cm for later transfer and assistance)? | Don’t Know |
| D.7 Is the toilet seat located between 40 and 48cm from the floor? | Don’t Know |
| D.8 Is the distance from the edge of the toilet seat to the rear wall between 650 mm and 800 mm ? |  |
| D.9 Is a corner toilet is in place: Is the distance from the pan to the adjacent wall min. 25cm? Is the distance from the center line of toilet to the adjacent wall min. 45cm? | Don’t Know |
| D.10 Are grab rails provided on both side of the toilet at a distance of 30-35cm from the center of the toilet and a minimum distance of 40cm from the wall? | Don’t Know |
| D.11 Is there a foldable rail at a height of 20-30cm above the seat level on the side where a lateral transfer is possible? Is the length of the foldable rail overlapping the front edge of the toilet seat in between 10-25cm? | Don’t Know |
| D12 Is the washbasin located between 75cm and 85cm from the floor? | Don’t Know |
| D.13 Is there a clear space under the washbasin between 65cm and 70cm height and 20cm deep? | Don’t Know |
| D.14 Is the edge of the washbasin located within a distance of 35 to 60cm from the wall? | Don’t Know |
| D.15 Is the tap type mixer, lever or sensor? Is the tap located at maximum 30cm from the edge of the washbasin? | Don’t Know |

Comments:

Photos of Section D:

| E- EMERGENCY, EVACUATION, ALARMS | Answer |
| --- | --- |
| E.1 Is there a fire alarm system, do they have both flashing lights and audible signals? | Don’t Know |
| E.2 Are emergency doors and evacuation routes signalized? | Don’t Know |
| E.3 Is there an established evacuation plan in case of emergency? | Don’t Know |

Comments:

Photos of Section E:

Form Analysis